Comet Mining -- An Overview

From extropians-request@gnu.ai.mit.edu Mon Jul 19 22:31:02 1993

To: ExI-Essay@gnu.ai.mit.edu

From: szabo@techbook.com (Nick Szabo)

Subject: Comet Mining -- An Overview

Date: Mon, 19 Jul 1993 21:59:26 -0700 (PDT)

Comet Mining -- An Overview

Copyright 1993 by Nick Szabo

Permission to redistribute w/attribution granted

-- Introduction --

A relatively unexplored area of space development, comet mining, may

play a central role in cracking open the space frontier.  Volatile

extraction, considered by this author and others[1] to be an important

near-term catalyst for large-scale space industrialization, involves

the delivery and processing of volatile ice (water, methane, ammonia,

etc.) delivered via ice rocket from Jupiter-family comets,  which

have elliptical orbits between Earth and Jupiter.  A visual presentation

and marketing analysis were given in [2], technical presentations can be

found in [3-4], and various technical and marketing issues have been

discussed in [1].  Government [4,10] and commercial [9] organizations

are starting to sponsor research in this area.  This paper attempts to

unite this information into a comprehensive introduction to comet mining.

-- Comets, the Jupiter Family, and the Need for Volatiles --

The only hard and fast rule to distinguish between comets and

asteroids is that comets have been seen to outgas.

Several asteroids have recently been reclassified as comets

when outgassing was discovered, eg Charon.  The outgassing

indicates active sublimating volatiles.  Asteroids either

lack free volatiles or they never get warm enough to outgas.

Different volatiles (H2O, CO2, CH4, NH3, etc.) outgas

at different temperatures.  Objects in circular orbits

tend to reach an equilibrium temperature where outgassing

stops; all known comets are in elliptical or chaotic orbits

where temperatures change over the course of the orbit.

By far the largest proportion of materials used by most

processing industries are volatiles and organics.  This is true

for Earth industry -- oil for energy, wood for structure, plants

and animals for food, and vast amounts of water and air

for nearly industrial processes, which we often take for granted.  In

space, SSF expendables consist overwhelmingly of volatiles: air, water,

propellant, etc.  The most advanced theoretical technology,

worked out in detail by K. Eric Drexler[8] relies

primarily on volatile and organic materials, especially carbon.  The

most promising microgravity industry, pharmaceuticals, would be

dominated by volatiles and organics.  By far the greatest bulk

of near-raw materials launched from Earth into space are volatile

propellants.  Even metals extraction and refining industries rely on

much larger amounts of air and water than they produce in metal product,

and it is quite a leap to assume we can eliminate this dependency

without costly R&D efforts and while maintaining reasonable levels

of efficiency and thruput.

That’s bad news, given that practically all native

materials work today has, for historical and political

reasons, focused on the dry Moon, and to a lesser extent

dry asteroids.  The good news is that volatile

extraction from ice is much easier than trying to

split oxidized metals into oxygen and metal (not to

mention trying to capture solar wind particles for

hydrogen, carbon, nitrogen, etc.).  Comet ice, full of

a rich diversity of water, nitrogen, and organic compounds,

is readily available.

The best currently known targets for volatile extraction

are the Jupiter-family comets.  These have been tossed into

the inner solar system by Jupiter, into highly elliptical orbits,

with perihelions as low as Mercury’s orbit (0.4 AU) and aphelions

near Jupiter’s.  Influenced by the inner planets, the orbits slowly

circularize.  Most of the perihelions are near 1 AU, although

to some extent that’s observational selection; we’re less

likely to see objects with perihelions at 2 AU, and ice there

sublimates much more slowly.

The comets live very short active lives on a geological timescale,

100,000’s to millions of years, but Jupiter’s gravity well

is quite hungry and continually replenishes the supply when

comets wander its way from the Kuiper Belt or Oort Cloud.

Some of these visitors get a rought welcome; we recently saw a

comet calve into dozens of pieces as it swung in too close to

Jupiter.  Caught in orbit around Jupiter, these pieces, the

largest c. 10 km diameter, are projected to collide with the gas

giant (on the far side, alas) in 1994[7].

Most such comets are whipped into heliocentric orbit (ie orbit

around the sun), with aphelion at Jupiter and perihelion

in the inner solar system.  Over the comet’s lifetime this orbit continues

to circularize until one of the following happens: (1) all the

volatiles exposed to the sun bake out, and the comet turns into an

asteroid, or (2) perihelion increases, and volatiles

remain frozen during the entire orbit, again turning the

comet into an asteroid.  For this, reason, many scientists

believe that many earth-crossing asteroids, especially types

C and D, may be old Jupiter-family comets, and some may still

contain frozen volatiles.  We know that many earth-crossing

asteroids contain water, and perhaps ammonia, locked to the regolith by

hydration.  This is harder to extract.  We should consider

these closer objects as alternate targets for volatile extraction,

comparing the tradeoff in transport and equipment costs.

Thousands of earth-crossing asteroids are believed to exist

with round-trip delta-v’s from Earth orbit lower than the

lunar surface.   Jupiter-family comets take somewhat more energy

to get to; escape + 8 km/s one way for good windows, shaving off

a few km/s on the way there and/or the way back if we

can use Venus, Earth, or Mars for gravity assist [1,3,5].

With a slightly more sophisticated mission we can manufacture

aerobrakes by sintering comet dust, eliminating most of the

return delta-v [6].  A speculative method called

“cometary aerobraking” has also been proposed [1].

-- Ice Rockets --

The industrial flexibility of volatiles makes its first

and most dramatic impact in deep space transportation technology.

We can combine easily extracted native thermal propellants with a tankless

rocket design, eliminating the need to launch either propellant,

tanks, or heavy electric powerplants from Earth.  The specific transport

technology that embodies this principle has been dubbed the “ice

rocket” [1].  The ice rocket design [1,2,3] consists of a long cylinder

about the same size and shape as a Space Shuttle’s solid rocket booster,

but made out of ice and coated with a thin insulating paint.  To

this is attached a tiny thermal rocket, about the size of a fist,

and a  tiny nuclear reactor, or few square meters of mirror,

which concentrates sunlight on the rocket engine. The engine

slowly eats the ice, converting it into a high-velocity vapor

exhaust. The rocket engine is designed for minitiarization and

simplicity, so that dozens of them can be built and launched

on a small, commercial budget at launch costs not much

lower than today’s.   A larger nuclear-powered design is

presented in [4].

To mass-produce the ice rockets we melt cometary ice and

purify it with a centrifuge, in some designs combined with

an inflatable still.  We form the ice cylinder in two

steps. First we freeze a thin shell by wetting a large, cold

cylindrical form.  As this ice gets thicker, it freezes further

layers more slowly, so we start squirting small spheres

across a shaded vacuum.  These spheres freeze on the

outside, then accumulate on the inside of the cylinder.  Soon

the cylinder is filled with partly frozen water, which will

continue to freeze over several years while the rocket travels

towards its destination.

The icemaking equipment is the most important part of the

system.  It must  produce a very high ratio of  ice mass to

equipment mass (aka mass thruput ratio (MTR): output product

per year divided by equipment mass launched from Earth).

It must be automated and reliable; think of a tiny auto-maintained

sewage treatment plant. Other parts of the comet (organics, dirt,

etc.)  can be gathered and attached as payload.   The cylinder is

then attached to the small rocket engine, whose tiny thrust over the

course of two or three years delivers the payload to a variety of

destinations: orbits around the Earth, Jupiter, or Mars, the surface

of Earth’s Moon, or to asteroids. To get to high Earth orbit we must

exhaust about 90% of the ice, or 80% if we take a  couple

extra years to use a gravity assist. (See [5] for patched-conic

math to compute such trajectories, and [6] for safety issues

involved in using Earth for gravity assist and aerobraking

of various varieties and sizes of payloads).  We might also find hidden

in some Earth-crossing asteroids, in Martian moons, or at the

lunar poles, in which case more than 10% can be obtained.

If the output of the icemaking equipment is high, even 10% of

the original mass can be orders of magnitude cheaper than

launching stuff from Earth.  This allows bootstrapping: the

cheap ice can be used to propel more equipment out to the

comets, which can return more ice to Earth orbit, etc.  Today

the cost of propellant in Clarke orbit, the most important

commercial orbit, is fifty thousand dollars per kilogram.  The

first native ice mission might reduce this to a hundred dollars,

and to a few cents after two or three bootstrapping cycles.

Furthermore, we can deliver volatiles not just to Earth orbit,

but anywhere else in the inner solar system.  A volatile

dump around Mars can slice an order of magnitude off the

propellant needed to launch a large-scale mission to Mars.

Comet volatiles are synergistic with lunar operations, adding

the missing elements needed to make lunar exploration or

industry productive.

Once the volatiles and organics have been separated, they are

fed to a series of chemical microreactors and converted to

essential  nutrients and construction materials for

greenhouses.  Greenhouses can be made in a very simple,

automated fashion, for example by pumping air

into liquid polymer spheres which are then solidified and filled

with nutrients and trellises for the crop.  The crops grow not

only pharmaceuticals, but also fiber and resins to provide structural

strength for further greenhouses, and genetically engineered

enzymes are extracted and used in the chemical

microreactors.  The greenhouses contain a low pressure (c.

1/5 atm), CO2-rich atmosphere to facilitate the growth of

genetically modified fiber plants while keeping the engineering

task of building the pressure vessels minimal.

Methane, ethane, and several other hydrocarbons have been seen in

varying abundance (<1% to 5% for methane) in comets.  If you want

to get rich in 2020, design a system to extract the methane from

the water & ammonia ice and the gravel/muck of comets, perhaps

manipulating a large gas/plasma interface (cf. comet tail dynamics).

A refinement of the ice rocket manufacturing process

is a 3D printer to produce very large structures.  Shoots droplets of

several kinds of materials following a digital pattern.  For example a

high-temp-sublimating ice, a low-temp-sublimating ice, and a

ceramic slurry.  The target object forms on a very large, cold

radiator in the shade.  The goal is to have the particles mostly

freeze before they impact the target, but nevertheless stick to and

accumulate on precise points on the target (in 3-space, layer

by layer) without too much splatter.  If the target itself must

freeze the droplets we run into heat conduction problems pretty

soon, and the target object couldn’t get very thick.   The

low-sublimating-point ice allows hidden surfaces to be “etched” by

sublimation when the structure is rewarmed, provided there are

escape holes.

-- Open Issues --

Many engineering tasks need to be undertaken to make comet

mining a reality:

* Simple processes to create 1/5 atm pressure vessels from cometary

ice & tar, including greenhouse windows

* elaborate/refine 3d printer designs

* variable gravity bolo

* minimize processes requiring gravity

* Gas/plasma separation processes

* plant automation (maintenence, etc.) -- this may be the

primary technological bottleneck

* Nitrogenous fertilizer from ammonia

* Carbon -> CaC -> acetylene vs. syngas (CO + H2) -> hydrocarbons

via Fischer-Tropsch

* Phenolic resins vs. urea-formaldehyde/fiberboard

* Polyethylene, alpha-olefins, polystyrene, PVC

Among cometary science to do:

* Cometary sources of phosphate, potassium, and halogens

* Detailed analysis of comet surfaces needed to optimize

choices and enable autonomous operations

Among biotechnology to do:

* Detailed design for “self-reproducing” greenhouses

* Fiber source that facilitates automated processing

Also of great interest are low pressure & plasma manufacturing processes,

especially those that can be greatly scaled up in space.

We would like to manufacture variety of products from

cometary and asteroidal material, including

but not limited to:

* paints (for spacecraft thermal control)

* polymer structures and coatings

* aluminization of polymer surfaces (very large reflectors/solar energy)

* Inconel or silicon carbide frit (for rocket motor)

* radiation shielding

* propellant: thermal, mono- and bipropellant chemical, and ion/MPD

* pressure vessels

* pipes

* single-isotope diamond & fullerenes (C12, C13, C14)

* solar wind isotope separation (eg He3)

* etc. (your ideas go here)

Greenhouse Bootstrapping

The known Jupiter-family comets with lowest-eccentricity may get above

0C at their surface long enough to create an extensive plant growth.

Don’t need to move them.  Moving cometary materials to a warm circular

orbit via gravity assist and ice rocket is desirable for some

markets, but it’s not necessary to start out the operation if we

have a sufficiently flexible biosystem that can

colonize the comet.  There’s also the possibility of chemosynthetic

life to live off the energy frozen in the comet as free radicals,

in which case it doesn’t matter how far from the sun we are, but

that’s speculative so I’ll skip it for now.

A bigger problem is maintaining sufficient internal pressure

to keep water above the triple point (otherwise it sublimates

straight to gas, like dry ice).  One way is to go back to

the bag, except use it as a greenhouse instead of a still.

(Or in addition to a still -- plants exhale pure O2 and

H2O which can be extracted).  A second way, with much higher

MTR (mass thruput ratio), is to have the plants grow their own pressure

vessel, some kind of strong cellulose fiber bound together with a resin

secretion.  It need only hold a CO2-rich atmosphere at < 1/4

atm.

This presents a chicken-and-egg (or more

properly plant-and-seed :-) problem, so we probably would use

bags for the first perihelion.  The mass thruput ration of

this scheme becomes astronomical (pardon the Saganism :-), reaching

millions/year by the second decade of operation, far outstripping the

time cost of money, and all that from just one rocket payload full of

plastic bags filled with seeds.  This makes it highly economically

attractive, even with only a minute fraction of the greenhouse products

being separated (eg by electrophoresis) into pharmaceuticals, etc. and

sent back groundside, or materials being processed and used as propellant,

shielding, structure, life support, etc. in interplanetary or Clarke

orbit, or on the Moon, etc.

Going from native comet goop to pure water, and resin/fiber

for the pressure vessel, is an interesting problem in metabolic

engineering, which combines genetic engineering with an in-depth

knowledge of plant metabolism, in order to optimize certain growth

features (in this case cellulose and resin output, and resistance

to vacuum conditions).  I’d probably started with a fast-growing fiber

plant like hemp or jute or kudzu, and throw in some genes from

deep-salt-lake creatures that maintain an active water gradient across

their outer membrane, which should also be pretty good for vacuuum

protection.  Extensive testing/breeding in groundside vacuum

chambers and micrograv testing in low Earth orbit would be in

order.

-- Conclusion --

Extraction and processing of volatiles from the Jupiter-family comets,

combined with the crude but effective technology of

ice rockets, present a wide variety of new possibilities along

the path from our current small scale space operations to large-scale

space industrialization.  Native volatiles can be processed to

supply current space operations, while making possible new

industries with low up-front investment.  Bootstrapping

of transporation with native ice rockets and industry with

chemical microreactors and self-reproducing greenhouses blazes a

wide path along fertile territory, leading to the technological

and economic resources for large-scale space industry and space

colonization.

References

[1] Szabo, various articles posted to sci.space on native volatile

extraction and processing, 1988-present.  See also articles

by Paul Dietz, Gary Coffman, Phil Fraering, and others.

[2] Szabo, “Comet Mining”, a presentation to

Seattle Lunar Group, Feb. 1992 (visual

presentation & initial marketing analysis)

[3] Szabo, “Some Issues in Comet Mining”, May 1992,

unpublished (technical overview)

[4] Zuppero, et. al., in _10th Symposium on Space Nuclear Power

and Propulsion_ (AIP Conf. Proc. 271, 1993)

[5] Sauer, “Optimization of Interplanetary Trajectories with

Unpowered Planetary Swingbys”, AAS 87-424, pg. 253

[6] Szabo, “Safety of flyby & aerobraking for large payloads at Earth”,

sci.space Message-ID: <1992Sep28.061517.1316@techbook.com>

Mon, 28 Sep 1992

[7] Baalke, “Comet Shoemaker-Levy, Possible Collision With Jupiter in 1994”,

sci.astro Message-ID: <25MAY199322260259@kelvin.jpl.nasa.gov>,

and subsequent discussion.

[8] Drexler, _Nanosystems_, John Wiley & Sons 1992

[9] Brian Thill, Boeing Corporation, personal communications

[10] Tony Zuppero, U.S. Department of Energy, personal communications