Safety of Flyby and Aerobraking of Large Payloads at Earth

From extropians-request@gnu.ai.mit.edu Mon Sep  6 02:04:27 1993

To: ExI-Essay@gnu.ai.mit.edu

From: szabo@netcom.com (Nick Szabo)

Subject: Safety of Flyby and Aerobraking of Large Payloads at Earth

X-Original-To: exi-essay@gnu.ai.mit.edu

Date: Sun, 5 Sep 93 23:36:09 PDT

Safety of Flyby and Aerobraking of Large Payloads at Earth

copyright 1993 by Nick Szabo

permission granted to copy without alteration

We’ve often heard suggestions for capturing asteroids

into earth orbit, and for using earth as a gravity or

atmosphere brake in order to slow down large chunks

of incoming material.  For some these images bring visions of

wealth; for others visions of destruction. This article will

explore the costs and benefits of using the earth for gravity

assist and aerobraking of large payloads, and suggest some policies

for taking advantage of these benefits without harming the earth.

Orbiting planets contain enormous amounts of kinetic energy, and

tiny fractions of this energy can be tapped by spacecraft

to great advantage. The two mostly widely used methods are gravity

assist and aerobraking.

Here’s how a gravity assist works. The planet is moving,

so there’s our energy source. The slingshot can be computed

with the patched-conics approximation.  If we do a Hohmann

ellipse to the planet in the inertial frame, the trajectory

is a hyperbola in the frame of the planet.  The energy of

the vehicle is the same at symmetric points on opposite sides

of the hyperbola in the planet frame.  If we exit the rendezvous

moving in the same direction as the planet, we gain velocity

in that direction in the inertial frame.  If we exit the

rendezvous moving opposite the direction of the planet, we lose

inertial velocity.  Either gaining or losing velocity can

be useful, depending on where we’re going.

Aerobraking is simpler to understand.  In layman’s terms,

the air slows the spacecraft down, just like wind

resistance slows down a bicyclist.  In orbital mechanics

terms, the spacecraft exchanges momentum with the particles

in the atmosphere.  An interesting variant, called cometary

aerobraking, vaporizes a piece of ice a split second before

it intercepts the spacecraft at high velocity.  The spacecraft

uses the temporary cloud of gas to aerobrake, as if it were

a planetary atmosphere.

All these maneuvers allow us to tap into the energy

already stored in the orbits of the planets and minor

planets.  They can greatly reduce the mass of propellant

and tank needed for a mission; in the case of Galileo

the Venus-Earth-Earth-Jupiter trajectory saved it from

being cancelled when it had to substitute a smaller

upper stage for the powerful Centaur.

Very large payloads can benefit from these trajectories

for the same reason, especially at earth, which raises

the important question of safety.  We cannot tolerate

bringing a dinosaur-killer sized asteroid anywhere near

earth, or coming towards earth near an intercept trajectory.

Reentry of c. 100 ton Shuttles is safely performed and

tolerated towards inhabited areas, and natural fireballs

and meteorites massing several tons each hit the earth

harmlessly every year.  Somewhere between these two extremes,

we need to figure out the margins of safety and enforce them.

We’ll look at three techniques for using earth to change the orbital

trajectory of objects:

* fast aerobraking (eg Shuttle, Apollo)

* slow aerobraking (eg Hiten)

* gravity assist (eg Galileo)

All three of these can play important roles in reducing the costs of

capturing space materials from comet fragments and asteroids into

various earth orbits.  The delta-v savings are can be up to an order

of magnitude for gravity flyby, and up to two orders of magnitude for

aerobraking, for returning material from the near-earth asteroids

to earth.  Fast and slow aerobraking are also useful for inserting lunar

payloads into low earth orbit.

For gravity assist, the following data need to be considered:

* what is the margin of error due to fringe atmospheric density,

gravity anomaly and trajectory measurement error?

* how quickly and precisely can the on-board engines compensate for

trajectory errors directly before and during the flyby?

* can the operation be timed so that a worst-case error will cause

reentry over an uninhabited area (eg the ocean)?

* how much material on board is strongly toxic or radioactive?

* what is the worst-case scenario for the mass, composition, and

worst-case error trajectory of the payload?

For slow aerobraking, we must also consider the above points, paying

close attention to the fringe atmospheric density, since that is what

we are using to change the trajectory.

For fast aerobraking, we need to pay very close attention to upper

atmospheric density at all levels.  The error margins are much less.

Unless the worst-case scenario is trivial or the spacecraft is

well-controlled aerodynamically, fast aerobraking is much more

dangerous than slow aerobraking or gravity flyby.

A natural example of fast aerobraking is the large meteor, or

fireball.  Chyba[1] calculates burst diameter and yield, and concludes

that non-ferrous objects of diameter less that 50m typically detonate

too high to be dangerous.

Future military use of space might include constellations of heavy

kinetic engery vehicles, or javelins, manufactured from asteroid material

for the specific purpose of penentrating Earth’s atmosphere intact

and destroying the target below.  For commercial projects, we should

use the above data to determine whether is ethically and politically

tolerable, and whether it can be insured.

For the sake of discussion, I make the following initial proposed rules

of thumb:

gravity assist & slow aerobrake

* <.0001% chance of trajectory error sufficiently large for reentry

* if reentry, >98% chance it will occur over & towards

uninhabited area

* payload mass limits:

solid metallic materials: 5,000 tons (no piece > 1 ton)

stony materials: 10,000 tons (no piece > 5 tons)

carbonaceous materials or loose regolith: 30,000 tons

volatile ices w/pores: 50,000 tons

strongly toxic or radioactive material:

(varies by material; 1 ton typical)

The shape and attitude of the container also play a major role.

For example a long, thin cylinder will be dispersed more widely

than a sphere if it hits the atmosphere sideways instead of

headlong.

fast aerobrake:

* aerobrake must contain control surfaces sufficient to

give <.001% chance of reentry due to error

* if reentry, >98% chance it will occur over & towards

uninhabited area

* no strongly toxic or nuclear materials on board

* mass limits:

solid metallic materials: 200 tons

all other materials: 400 tons (note: ceramic heat shield will

be significant % of mass for any payload)

Significant amounts of simulation, study of real-life artificial

and natural reentries, and benefit/risk analysis need to go into

determining the actual safety margins.  Calculations like those

done by Christopher Chyba[1] and Zdenek Sekanina at JPL to predict the

ability of asteroidal and cometary material to penetrate the earth’s

atmosphere need to be perfected.

Privately financed insurance with unlimited liability

should be required for all such payloads.  If no insurance

company is willing to underwrite the risk it is a good sign

for the public that the maneuver is too risky and should be

not be tolerated.  On the other hand if the insurance industry

volunteers to take on the risk, this is a good sign that the

risks, financial and physical, are minimal, and that the payoff

directly to the companies, and indirectly to mankind as a whole,

are well worth it.

[1] _Nature_ (v.363, no.6431, 24-June-1993):

``Explosions of small Spacewatch object in the Earth’s atmosphere’’

Christopher F. Chyba, pp.701--703;